STANDARD OPERATING PROCEDURE

Fill out the form completely, print and place it with your laboratory-specific Chemical Hygiene Plan.  All personnel who are subject to the requirements of this SOP must review and sign the training record on the back of the SOP.

Materic Company: Monneah’s Engineered Materials	Date when SOP was written: 6/28/24

SOP Prepared by:  Aalliyeh Clinton

SOP Reviewed and Approved by (name/signature):

Approval Date:

Principal Investigator: Steve Farias

Laboratory Phone:					Office Phone (For PI):

Emergency Contact(s) (name/phone #): 911

Primary contact: Name of laboratory personnel supervising the procedures

Secondary contact(s): Stephen Farias – 484-802-7993, Kelli Booth – 601-818-0612

Location(s) covered by this SOP (building/room number(s)):

Section 1:  		 Process	 Hazardous Chemical		 Hazard Class

- PROCESS ABSTRACT

This experiment aims to evaluate the chemical permeation resistance of cotton textile specimens when exposed to acetic weak acid. The testing follows ASTM F903-18 guidelines, focusing on Procedures A and B without the application of hydrostatic pressure.

Cotton and Tyvek textile samples, are cut to a standardized size of 5 cm x 5 cm, are subjected to continuous contact with acetic acid. The primary objective is to assess the penetration resistance and identify any degradation over a defined exposure period.

The results of this experiment will provide valuable insights into the suitability of cotton vs Tyvek textiles for protective clothing applications in environments where exposure to these chemicals is a concern. Observations and data collected will include the time to visible penetration and any material degradation, contributing to the understanding of the protective efficacy of cotton textiles against various chemical hazards.

- POTENTIAL HAZARDS

Hazardous Chemicals:

Weak Acid:

Acetic Acid (5%) for 5% acetic acid, white vinegar would be perfect for the job

Health Hazards: These can irritate the skin, eyes, and respiratory system.

Flammability: Acetic acid is flammable in higher concentrations, but at 5%, the risk is significantly reduced. However, it can still enhance the combustion of other materials.

Chemical Reactivity: May react with strong oxidizing agents and bases, releasing heat and potentially hazardous fumes.

PERSONAL PROTECTIVE EQUIPMENT (PPE)

Discuss the personal protective equipment and hygiene practices used with each process, class of chemicals, or individual chemicals.

  - - Gloves: Nitrile, butyl rubber gloves, or polyvinyl chloride gloves would be best option when handling acetic acid.

- Lab Coat: Chemical-resistant lab coat

- Eye Protection: Safety goggles or face shield

- ENGINEERING/VENTILATION CONTROLS

Fume Hood: All experiments will be conducted inside MTI FH-36 fume hood to ensure proper ventilation.

- SPECIAL HANDLING PROCEDURES AND STORAGE REQUIREMENTS

  - Storage:

- Location: Store in a cool, well-ventilated area away from incompatible substances

Container: Keep tightly closed in a glass bottle/jar-resistant container.

  - Transportation:

- Use secondary glass containment for transporting chemicals.

- Transport small quantities to avoid spills.

- SPILL AND INCIDENT PROCEDURES

Small Spill Cleanup (<500 mL)

Neutralize: Use a neutralizing agent, such as sodium bicarbonate (baking soda), to neutralize the acid. Sprinkle it over the spill, and ensure it covers the entire area. It will react and fizz as the acid is neutralized.

Absorb: Once neutralized, use absorbent materials like spill pads, paper towels, or vermiculite to soak up the liquid. Ensure all liquid is absorbed before proceeding.

Dispose: Place the used absorbent materials in a sealed, labeled chemical waste bag. These materials should be treated as hazardous waste. Once it is neutralized and absorbed

Decontaminate: Wash the affected area thoroughly with water and soap or a mild cleaning solution. Ensure no residue remains.

Ventilate: Continue to ventilate the area until all fumes have dissipated.

Large Spill Cleanup (>500 mL)

Evacuate: Evacuate all personnel from the immediate area if the spill is large or if fumes are overwhelming.

Call for Help: Notify the emergency response team or safety officer immediately. Large spill mayrequire professional handling.

Ventilate: Ensure fume hoods are operational or open windows to disperse vapors while awaiting professional responders.

Contain the Spill (if safe):

Use spill containment booms or absorbent pads to prevent the spread of liquid to other areas.

Avoid entering the spill area if the fumes are too strong or there is risk of slipping.

Incident Reporting

Report the Incident: Immediately report the spill and cleanup procedures to the lab supervisor or safety officer.

DECONTAMINATION PROCEDURES

Clean all equipment and glassware with appropriate solvents and neutralizing agents.

- Equipment: Clean with a solution of sodium bicarbonate or dilute ammonia, followed by water.

Skin: Wash thoroughly with soap and water.

- WASTE DISPOSAL

1. Materials

Neutralizing Agent: Sodium bicarbonate (baking soda) is commonly used.

Beaker or Container: Use a glass or chemical-resistant container for the neutralization.

pH Strips or Meter: To check that the solution is neutralized.

Absorbent Material: Paper towels or spill pads for cleaning up any potential splashes.

2. Slowly Add Sodium Bicarbonate

Prepare the Container: Place the acetic acid in a glass beaker.

Add Sodium Bicarbonate: Slowly add small amounts of sodium bicarbonate to the acetic acid. You’ll notice fizzing and bubbling as the acid reacts with the sodium bicarbonate, releasing carbon dioxide gas.

Important: Add the sodium bicarbonate gradually to avoid overflow from excessive bubbling.

3. Monitor the Reaction

Allow the reaction to proceed until the fizzing subsides. Continue to add small amounts of sodium bicarbonate until no more bubbles form.

Gently swirl the container to ensure even mixing.

4. Check the pH

Use pH strips or a pH meter to test the solution. The solution should have a neutral pH (around 7) before disposal.

If the pH is still acidic (below 7), continue adding small amounts of sodium bicarbonate and retest.

5. Dispose of the Neutralized Solution

Drain Disposal: If local regulations allow, the neutralized solution (pH 6-8) can be safely poured down the drain with plenty of water. Ensure you’re following your institution’s waste disposal guidelines.

Run water for 1-2 minutes before and after pouring the neutralized solution to dilute it further.

6. Clean Up

Wipe down any surfaces or equipment used with water and detergent.

Dispose of any paper towels or absorbent materials used in the cleanup according to lab protocols, ensuring they don’t contain neutralized acid.

8. Record Disposal

Document the amount of acetic acid neutralized and disposed of, along with the pH level reached, in your lab’s waste log

- PRIOR APPROVAL/REVIEW REQUIRED

Any deviation from the specified procedure must be approved by the PI.

- Review required for scale-ups, changes in chemical concentrations, or use of new chemicals.

- DESIGNATED AREA

The MTI FH-36 fume hood is designated for all chemical permeation testing procedure.

- SAFETY DATA SHEETS AND OTHER RESOURCES

A binder with Safety Data Sheets (SDS) for all chemicals used.

- DETAILED PROTOCOL

Procedure and Safety Requirements for Testing Permeation According to ASTM F903-18

Materials and Equipment

- Test cell as specified in ASTM F903-18

- Sealing materials (parafilm or silicon rubber gasket)

- Collection medium (isopropanol) What is this used for?

- Analytical equipment capable of measuring concentration at or below 0.05 μg/cm²/min

- Appropriate personal protective equipment (PPE) for handling chemicals

-Specimen Preparation

1. Cutting Specimens: Cut textile specimens to 5cm x 5cm in the test method, ensuring they are flat and planar.

2. Sealing Specimens: Validate and document the sealing method to prevent peripheral leakage.

Experimental Procedure

Materials and Equipment:

Test Specimen: 5x5 cm square cut from the protective clothing material.

Test Cell: 400 mL glass beaker (suitable for holding the 5x5 cm specimen).

Sealing Material: Parafilm or rubber gasket to ensure airtight sealing of the specimen in the test cell.

Test Chemical: The specific liquid to be tested (e.g., acetic acid, as specified).

Collection Medium: An appropriate solvent, such as isopropanol, to capture the permeated chemical.

Analytical Instrument: High-Performance Liquid Chromatography (HPLC) or Gas Chromatography-Mass Spectrometry (GC-MS) for measuring chemical concentration at or below 0.05 μg/cm²/min.( if not available spectrophotometry) We don’t have either of the equipment

Timer: For recording exposure and penetration times.

Procedure:

- Setup:

  - Place the 5x5 cm specimen flat over the opening of the 400 mL glass beaker.

  - Seal the specimen tightly to the beaker using parafilm or a silicon rubber gasket, ensuring there is no peripheral leakage. The seal must be secure to prevent false penetration results due to improper sealing.

- Chemical Exposure:

  - Pour the test chemical into the beaker until it covers the entire surface area of the specimen.

  - Start the timer and expose the specimen to the test chemical for 10 minutes. Maintain the chemical in continuous contact with the specimen during this period.

- Observation and Recording:

  - Monitor the specimen continuously during the 10-minute exposure.

  - Observe for any visible penetration of the chemical through the specimen. This more of a comment: what and how do you observe penetration?

  - Record the time at which penetration occurs, if it does, and measure the permeation rate at 0.1 μg/cm²/min using the analytical instrument.

- Collection and Analysis:

  - After the exposure period, carefully collect any permeated liquid in the collection medium (isopropanol) from the beaker.

  - Use HPLC or GC-MS to analyze the concentration of the permeated chemical in the collection medium. Ensure the detection limits of the instrument can measure concentrations at or below 0.05 μg/cm²/min.

- Report Findings:

  - Document the breakthrough time (the time at which penetration is first observed).

  - Report the permeation rate at 0.1 μg/cm²/min. If testing is continued, also report the permeation rate at 1.0 μg/cm²/min.

  - Include any observations of degradation or other relevant factors in the final report.